The actual problem

Necessity to implement and use algorithms of localization and tracking of Whales to avoid the collision between them and the SHIPS

[Statistics of collisions, number of whales, impact of the collisions in the animals and people]

Historical records suggest that ship strikes fatal to whales first occurred late in the 1800s as ships began to reach speeds of 13–15 kn, remained infrequent until about 1950, and then increased during the 1950s–1970s as the number and speed of ships increased. Of 11 species known to be hit by ships, fin whales (Balaenoptera physalus) are struck most frequently; right whales (Eubalaena glacialis and E. australis ), humpback whales (Megaptera novaeangliae ), sperm whales (Physeter catodon ), and gray whales (Eschrichtius robustus ) are hit commonly. In some areas, one-third of all fin whale and right whale strandings appear to involve ship strikes. To assess contributing factors, we compiled descriptions of 58 collisions. They indicate that all sizes and types of vessels can hit whales; most lethal or severe injuries are caused by ships 80 m or longer; whales usually are not seen beforehand or are seen too late to be avoided; and most lethal or severe injuries involve ships travelling 14 kn or faster. Ship strikes can significantly affect small populations of whales, such as northern right whales in the western North Atlantic. In areas where special caution is needed to avoid such events, measures to reduce the vessel speed below 14 kn may be beneficial.

Ship collisions with motorized vessels appear to have begun late in the 1800s and to have remained relatively infrequent until the 1950s. From the 1950s through the 1970s they increased to approach current levels. In some areas ship strikes are now responsible for a substantial proportion of large-whale strandings.

Cetacean species are very susceptible to serious injury and mortality from vessels.  Areas of high marine traffic can pose a lethal threat to these animals, especially in “bottlenecks” where both whale and ship densities are concentrated (Williams and O’Hara, 2010).  Unfortunately many incidents of ship strike around the coast go unnoticed or unreported, and this makes it difficult to understand the scope of the problem.  For instance, large vessels such as cruise ships may not perceive the impact of a collision – even with a large whale species.  In fact in 1999 and again 2009, a fin whale was struck by a cruise ship in BC waters.  Jensen and Silber (2003) reported that fin whales are the most frequently struck large cetacean, at nearly twice that of the next most commonly struck species – humpback whales.  Injury and death as a result of ship strikes are significant threats to recovering populations of marine mammals and also has the potential to damage smaller vessels and cause injury to passengers.

CANARIAS 1 DEATH

However, big ships are not the only boats responsible for cetacean collisions.  The summer of 2006 was an unfortunate season in BC as two killer whales, two humpback whales and a minke whale were seen either being struck by vessels or were observed with recent scars from collisions with vessels.  Both killer whales involved did not survive and one of the humpback whales has not been resighted since.

In Glacier Bay, along Alaska’s Pan Handle, an unusually high number of humpback whales were found dead in 2010.  The necropsies of two whales in particular found injuries consistent with high impact blunt force trauma, such as skull fractures.  In both cases vessel collision was determined as the cause of death (Neilson and Gabriele, 2010).

Fin whales, humpback whales, and killer whales may be the most common whales involved in vessel collisions, but researchers are starting to notice evidence of ship strike among smaller cetaceans as well.  In April 2011, Pacific white-sided dolphins were foraging in Howe Sound and several animals displayed the tell-tale scars and injuries inflicted from a boat.  One animal was observed with a severed dorsal fin and other dolphins were photographed with propeller scars on their backs.

WHAT YOU CAN DO

1. Observe the Be Whale Wise Guidelines
2. Slow down to less than 7 knots within 400 m of whales.
3. In areas of known cetacean activity, reduce speed and keep a lookout for signs of whales, dolphins and porpoise.  Remember- whales don’t always know where you are!

Results

The cause of death were attributed to ship strikes when they reported either (1) massive blunt impact trauma (e.g., fractures of heavy bones including skulls, jaws, or vertebrae) or apparent propeller wounds (i.e., deep slashes or cuts into blubber on the dorsal aspect, or (2) a dead whale on the bow of a ship

Of the living right whales for which good-quality photographs are available, 7% (12 of 168) had scars caused by ship strikes. An updated analysis (Knowlton and Kraus, in press) links ship strikes to 35% (15 of 43) of right whale deaths between 1970 and 1998, and to at least 47% (8 of 17) of their deaths from 1991 to 1998, a period when carcass recovery and necropsy efforts improved. Because there are only about 300 animals in the population (Knowlton et al. 1994, Caswell et al. 1999), ship strikes pose a serious threat to recovery and intensive management efforts have been undertaken in both the United States and Canada to reduce the number of vessel-related deaths (Marine Mammal Commission 1999).

Overall, 14% (58 of 407) of the records indicate vessel collisions as the known or possible cause of death

Large vessels

Velocity

6. Ship collisions probably have a negligible effect on the status and trend of most whale populations, but for very small populations or discrete groups, they may have a significant effect.

Track the animals in real time and their behaviour in determined areas

Known about the underwater environment (objects, animals)

Demonstration Mission Listening to the Deep-Ocean Environment (LIDO) algorithms for the automated real-time detection, classification and localization of cetaceans have been developed. Such Passive Acoustic Monitoring techniques have the potential to play a key role in cetaceans’ conservation for they allow a non-invasive study of their behaviour, a better knowledge of their population dynamics, and a better understanding of their dynamic relationship with their environment. This paper investigates the development of efficient and accurate techniques to be used as the basis of a localisation module for an automated real-time Passive Acoustic Monitoring system. An opportunity to assess the capabilities of the developed localisation modules was given by recordings collected with a bottom. A well-known class of methods for acoustic source localisation is based on time differences of arrival (TDOA). Its capabilities have shown to be useful even in adverse situations (i.e., few sensors, high noise levels and/or poor calibration).

NOT ONLY WHALES, ALSO OTHER ANIMALS

THE WAY TO REDUCE OUR IMPACT IN THEIR LIFES

The solution

- Estimate the whale localization by TDOA

    - What is TDOA?

    - Sensors used?

    - Necessity of noise reduction?

    - Actual algorithms?

time difference of arrival (TDOA) is commonly used in civil and military surveillance applications to accurately locate an aircraft, vehicle or stationary emitter by measuring the time difference of arrival (TDOA) of a signal from the emitter at three or more receiver sites. It locates an object by accurately computing the time difference of arrival (TDOA) of a signal emitted from the object to three or more receivers. It also refers to the case of locating a receiver by measuring the TDOA of a signal transmitted from three or more synchronized transmitters.

It requires mainly a two step method. In the first step, the time differences of arrivals of the signal of interest to the different sensors are measured. This can be done by crosscorrelation and peak detection of the sensors’ signals. Many methods have been presented that deal with enhancing the correlation result, among them [1],[2]. In the second step, the measured TDOAs are used to solve the hyperbolic system. Closed form algorithms and iterative algorithms can be found in [3],[4]. Kalman Filters are also widely used because of their robustness and high accuracy [5],[6].

TDOA endeavors to determine the travel time of a wavefront between two spatially separated receiving sensors, which is often of concern to a passive system such as passive sonars and microphone array systems. Although there exists intrinsic relationship between the TOA and TDOA estimation, their essential difference is literally profound. In the former case, the “clean” reference signal, that is, the transmitted signal, is known, such that the time delay estimate can be obtained based on a single sensor generally using the matched filter approach.

On the contrary, in the latter, no such explicit reference signal is available, and the delay estimate is often acquired by comparing the signals received at two (or more) spatially separated sensors. This paper deals with TDE, with its emphasis on the TDOA estimation The estimation of TDOA would be an easy task if the two received signals were merely a delayed and scaled version of each other. In reality, however, the source signal is generally immersed in ambient noise since we are living in a natural environment where the existence of noise is inevitable. Furthermore, each observation signal may contain multiple attenuated and delayed replicas of the source signal due to reflections from boundaries and objects. This multipath propagation effect introduces echoes and spectral distortions into the observation signal, termed as reverberation, which severely deteriorates the source signal. In addition, the source of the wavefront may also move from time to time, resulting in a changing time delay. All these factors make time delay estimation a complicated and challenging problem.

Over the past few decades, researchers have approached such a problem by exploiting different facets of the received signals. Numerous algorithms have been developed, and they can be categorized from the following points of view:

(i) the number of sources in the wavefield, that is, singlesource TDE techniques [5, 9] and the multiple-source TDE techniques [10, 11];

ii) how the propagation condition is modeled, that is, the ideal single-path propagation model [5], the multipath propagation model [12–14], and the reverberation

model [15–17];

(iii) what analysis tools are employed, for example, generalized cross-correlation (GCC) method [5, 18–22], higher-order-statistics-(HOS) based approaches [23, 24], and blind channel identification based algorithms [15, 25];

(iv) how the delay estimate is updated, that is, non-adaptive and adaptive approaches [26–30].

The improvements are achieved through three different ways:

The first is to incorporate some a priori knowledge about the distortion sources into the GCC method to ameliorate its performance.

The second is to use multiple (more than two) sensors and take advantage of the redundancy to enhance the delay estimate between the two selected sensors.

The third is to take into account of reverberation in the signal model and exploit the advanced system identification techniques to improve TDE.

Two or more sensors:  Two more efficient and there are algorithms good for the TDE.

Other with more sensors like: Fusion algorithm based on multiple sensor pairs ^Priori knolgede

Multichannel cross-correlation algorithm

Adaptive multichannel time delay estimation

cc,gcc,

LMS, AED,

Noise reduction

NOISE REDUCTION BECAUSE IN WATER THE SIGNAL IS HIGLY CORRELATED AND WITH A HIGHT LEVEL. WE NEED TO ELIMINATE THAT, because in the algorithms we will not be able to find the true delay because of noise.

- si es blanco te jode el conocer el channel

- si es correlado y quieres usar metodos de correlacion, siempre detectaras el ruido y no la señal

Therefore, we will use Noise algorithms to:

-Reduce noise and interference :   make signals more evident     symplify detection

- put signal into standarised form    easy to set standarised threshold    well SNR  low background noise level

- Benefits of TDOA vs Beamforming in the underwater environment.

- Why?

Particularities of underwater environment

A second class of methods, the space–time methods, originated in underwater applications such as sonar but reached its most significant achievements over the last 20 years in digital communications with recent progress in the treatment of broadband signals. Various broadband space–time methods were implemented and allowed to map the sound radiated during the detected clicks and to consequently localise both sperm whales and vessels. Hybrid methods were also developed which improved the robustness of space–time methods to noise and reverberation and reduced processing time. In most cases, the small variance obtained for these estimates lessened the necessity of additional statistical clustering. Even though not independently confirmed by sightings, the tracks derived in the proposed frame can be considered to be consistent with the known movements of sperm whales and vessels

There are two general groups of methods for the localization of sources : TDOA-based methods and Space-time methods (~ beamforming). In the second type of methods based on spatial filtering, only one step is required but those methods are more computationally demanding since they require to scan the whole space. The first type of methods is more general and deals with a very simple yet a key problem for signal processing and estimation theory: time-delay estimation. In these methods time-delay estimation is performed and then a time-space inversion constructs a position based on the estimated time-delays.

WE USE TDOA BECAUSE THE DISTANCE OF THE SENSORS. IF WE WOULD HAVE USED BEAMFORMING, WE had seen SPATIAL ALIASING

Large distance between sensors. It is more robust to bad calibration (amplitude, sensor position). If your array fits all the requirements for it, do beamforming, otherwise you have no choice but to use the TDOA approach.

simplicity, robustness to reverberation and absence of model assumptions. Main drawback: multi-source TDOA is difficult

This is the only feasible method

for this case, as beamforming cannot be used when distance

between sensors is large.

Beamformer

A beamformer is steered to all directions in order to compute the spatial power, assuming that the power from the source direction is high. So, directions that maximize the output power are the estimated locations.

Often the maximization function does not present strong global peaks

The methods with better performance are computational expensive

Most of these methods are not designed for arbitrary array geometries

Assume narrow-band signals, while audio is a broadband signa

Goals of the project (Enunciate)

- Implement with MATLAB and compare the performance of various time-delay estimators (CC, Generalized-Cross-Correlation (GCC), LMS-filter or the very interesting Adaptive-eigenvalue analysis) and noise reduction algorithms with both simulated and real underwater acoustic data.

- Advantages

- Disadvantages

- Peculiarities of each method

cc: The cross-correlation (CC) method is the most straightforward and the earliest developed TDE algorithm, which is formulated based on the single-path propagation model given in (1) with only two receivers, that is, N = 2. then the delay estimate with the CC method is obtained as the lag time that maximizes the cross-correlation function (CCF) between two observation signals, that is,

GCC: The generalized cross-correlation (GCC) algorithm can be treated as an improved version of the CC method. Not only does it unify various correlation-based algorithms into one general framework, but it also provides a mechanism to incorporate knowledge to improve the performance of TDE  Commonly used weighting functions include the constant weighting (in this case, the GCC becomes a frequency domain

implementation of the cross-correlation method shown in (9)), the smoothed coherence transform (SCOT) [44], the Roth processor [45], the Echart filter [5], the phase transform (PHAT), the maximum-likelihood (ML) processor [5], the Hassab-Boucher transform [18], and so forth. Combination of some of these functions is also reported in use [46].

CONS: NO ADAPTIVE!!  RUIDO

LMS: Different from the cross-correlation-based approaches, this algorithm achieves time delay by minimizing the mean-square error between x0[k] and a filtered (FIR

filter) version of x1[k], and the delay estimate is obtained as the lag time associated with the largest component of the FIR filter. estimated through

h[k + 1] = h[k] + μe[k]x1[k], (20)

where μ is a small positive adaptation step size. Given this estimate of h[k], the delay estimate can be determined as

τLMS = argmax IhI-M

Other adaptive algorithms [48] can also be used, which may lead to a better performance.

CONs: RUIDO!! Y computation

AED an adaptive eigenvalue decomposition (AED) algorithm was proposed to deal with TDE in room reverberant environment [15, 55]. Unlike the crosscorrelation- based methods, this algorithm first identifies the channel impulse responses from the source to the two sensors. The delay estimate is then determined by finding the direct paths from the two measured impulse responses. Apparently, this algorithm takes fully into account the reverberation effect during time delay estimation.

Cons:

Computation EL DOBLE QUE LMS

JODIDO EN EL RUIDO! pero no en el delay estimation

TIMEGAIN: set average amplitude to a standart level. Then, do the normalization of the signal by the averaged signal

PNR: Estimate noise by using the percentile value, then subtract the noise from the signal spectrogram

Spectral Subtraction: The same if we use percentile=50.

- Localization of the Whale from the Time delay Estimations obtained from the Simulations with our algorithms and real data.

In this final Section, we use the TDOAs estimated in the previous. Section from real hydrophones signals, for tracking a sperm whale on a 2D map. We develop a rudimentary multilateration system, on the basis of 2 TDOAs only.

Multilateration

At least two pairs of mics are required for multilateralization, i.e. 3 mics

Multilateration is a navigation technique based on the measurement of the difference in distance to two or more stations at known locations that broadcast signals at known times. Unlike measurements of absolute distance or angle, measuring the difference in distance results in an infinite number of locations that satisfy the measurement. When these possible locations are plotted, they form a hyperbolic curve. To locate the exact location along that curve, a second measurement is taken to a different pair of stations to produce a second curve, which intersects with the first. When the two are compared, a small number of possible locations are revealed, producing a "fix".

Multilateration is a common technique in radio navigation systems, where it is known as hyperbolic navigation. These systems are relatively easy to construct as there is no need for a common clock, and the difference in the signal timing can be measured visibly using an oscilloscope. This formed the basis of a number of widely used navigation systems starting in World War II with the British Gee system and several similar systems introduced over the next few decades. The introduction of the microprocessor greatly simplified operation, greatly increasing popularity during the 1980s. The most popular hyperbolic navigation system was LORAN-C, which was used around the world until the system was shut down in 2010. Other systems continue to be used, but the widespread use of satellite navigation systems like GPS have made these systems largely redundant.

Multilateration should not be confused with trilateration, which uses distances or absolute measurements of time-of-flight from three or more sites, or with triangulation, which uses the measurement of absolute angles. Both of these systems are also commonly used with radio navigation systems; trilateration is the basis of GPS.

Other: In trigonometry and geometry, triangulation is the process of determining the location of a point by measuring angles to it from known points at either end of a fixed baseline, rather than measuring distances to the point directly (trilateration). The point can then be fixed as the third point of a triangle with one known side and two known angles.

We first show the position of the hydrophones on the map (Fig. 6.27), and then plot the hyperbolas corresponding to each pair of TDOA, assuming the speed of sound in water is 1510 m/s. Notice we drop the first TDOA estimates, which the LMS algorithm used for converging, and only use 10 estimates for our plot.

For 2D multilateralization from intersection of loci we need to solve a system of two non-linear equations. The third TDOA may not have the same intersection

Other

Multiple TDOA/DOA estimates lead to a minimization problem of an over-determined error function E(s) of position s:

Direct recursive algorithms are possible (gradient descent) but present some problems: initial estimate, instability, local minimum...

Closed-form approximations are computationally practical and perform well in easy scenarios. However, difficulties in very high noise and reverberant scenarios and the inability to work on multi-source situations are the main drawbacks. Main methods: crossing lines (CL), steered-response power (SRP)

Deepen the goals

- Implement and compare algorithms (95% of the spent time)

First, read the papers about the different time-delay estimation algorithms in different environments and papers about Minke Whales and their kind of sounds.

Rankin_JASA05_Minke_Boing-----Minke whales paper

tde_ IN ROOM-------- all the algorithms performance in room

On using the LMS on tde---- LMS

shipstrike-------     Collisions between whales and ships

eigenvalue decomposition algorithm-----------AED vs GCC

Tde using LMS--------------------LMS

Collisions between vessels and whales

whale spotting ---------- TDE theory and code with Matlab

- Deal with real data

- Minke whales (History and curiosities of Minke whales and the sound characteristics)

During a recent cetacean survey of the U.S. waters surrounding the Hawaiian Islands, the probable source of the mysterious “boing” sound of the North Pacific Ocean was identified as a minke whale, Balaenoptera acutorostrata. Examination of boing vocalizations from three research surveys confirms previous work that identified two distinct boing vocalization types in the North Pacific. The eastern boing _n=22_ has a pulse repetition rate of 92 s−1 and a duration of 3.6 s and was found only east of 138°W. The central boing _n=106_ has a pulse repetition rate of 115 s−1 and a duration of approximately 2.6 s and was found only west of 135°W. Central boing vocalizations produced by a single source _n=84_ indicate that variation in repetition rate and duration of the calls of the individual were not significantly different than the variation among individuals of the same boing type. Despite a slight latitudinal overlap in the vocalizations, pulse repetition rates of the eastern and central boings were distinct.

The “boing” sound was first described by Wenz _1964_ from U.S. Navy submarine recordings made in the 1950s off San Diego, California, and Kaneohe, Hawaii. Despite much attention, the source of the sound has remained a mystery until now.

This paper details the events leading us to attribute the boing sound to the North Pacific minke whale and summarizes the characteristics of the boing vocalizations recorded during this particular encounter and during three research cruises in the North Pacific Ocean.

Boings were detected seasonally, from November through March, and had an estimated sound source level of 150 dB re 1 _Pa at 1 m _Thompson and Friedl, 1982_. Given this information, Thompson and Friedl _1982_ suggested that the sound source was likely a whale, but they did not speculate as to which species. The first suggestion that the boing may be produced by the minke whale _Balaenoptera acutorostrata_ was made by Gedamke et al. _2001_ based on the structural similarity of the boing and the sound produced by the dwarf minke whale in the Great Barrier Reef, Australia.

Boings were detected during three cetacean research surveys, the 1997 Sperm Whale Abundance and Population Structure Survey _SWAPS_, the 2002 HICEAS cruise, and the 2003 Stenella Abundance Research Survey _STAR_.

Boing vocalizations consist of a brief pulse followed by a long call that is both frequency modulated _FM_ and amplitude modulated _AM_ _Figs. 2 and 3_. Based on our measurements of 128 boings, the calls can be grouped into two distinct call types with nonoverlapping pulse repetition rates _Fig. 4, Table I_. Those with pulse repetition rates of 91–93 s−1 were all detected east of 138°W and match Wenz’s _1964_ description of the San Diego boing _Fig. 5_. Those with pulse repetition rates of 114–118 s−1 were all detected west of 135°W and matchWenz’s description of the

Hawaii boing _Fig. 5_.

The eastern boing has a significantly longer duration _mean =3.6 s, n=22_ than the central boing _mean=2.6 s, n=106_ Minke whales are the smallest of baleen whales and are typically encountered individually or in small groups of two or three. They have inconspicuous blows, and do not surface for extended periods of time. Additionally, the waters covered by these surveys are dominated by high sea states associated with the trade winds. Combined, these features reduce the probability

of sighting minke whales, and may explain the discrepancy between the low visual detection of minke whales and the high acoustic detection of boings. The swimming speed and direction of the minke whale during this encounter _sighting number 267_ was found to be nearly identical based on the visual and acoustic detections _5.7 km/h and 5.6 km/h, respectively_. This speed is reasonable, but higher than that found by other researchers

Our Real Data provided by Ludwig  : describe data where does it comes from, why is this case particular compared to the data we have used in class ( species, distance between sensors, duration od signals and distance between minke whales sounds compared to sperm whales, etc)

- Duration of the data-

- Number of sensors-

- Events taken into account

- True delay of the two events

- Sampling rate  (Enunciate the future problems with LMS and AED due to the Hight sampling rate, the problem is not really the sampling rate (it's better to have more samples !) but the distance between the sensors)-

Data and information is coming from PMRF,  an instrumented US Navy testing range located off the island of Kauai, Hawaii. Data were collected from seven bottom mounted hydrophones (4 to 5 m off the seafloor) in deep water (nominally 4,600 meters) approximately 45 km northwest of Kauai. The relative location of the hydrophones, their designations and filenames for the data files are as shown in Table 1.

With this dataset you’ll be able to make localization of minke whales (Balaenoptera acutorostrata) based on their boings sounds.. Beware that vocalizations from other species are also present,

The dataset is from the 27th of April 2009 at approximately 12:00 Hawaiian standard time. Thirty minutes of data from each hydrophone are provided as three files (approx. 10 minutes per file). The sampling rate of the recordings (windows PCM wav format) is 96 kHz with 16 bits resolution. The data are sampled simultaneously, so sample N from one file is the same relative time as sample N of a second file with the same filename. The file format is .wav in little-endian format.

Distance

Distance between sensors, 12-27 km. This leads to a difficulty to implement and work with LMS and AED due the big maximum time delay between sensors. This leads to have a big big filter length and added to a big sampling rate, its becomes a difficult time delay estimation with our computers due the computational calculs.

Events taken

We focus on two events of the first 10m of recording.  Around the minut 2:30 and the minut 8:50.  The treu delay varies, but looking into Audacity we have calculated it. It varies from 1s to 12 s between the seven sensors in the two events. The delay between the two events vary, this would explain the whale is moving.

Sperm

The difference with sperm whales, the sounds we have used in class, is that the sperm whales have a spectrogram with cliks, little impulses like deltas. En cambio, las minke whales had other sound, with more high frequency. The sounds are not clicks, and therefore we have more difficulties to calculate de tde.

sperm ::: The distance between most hydrophones is 5 nautical miles (about 7.5 km).

% We will work with the sounds received by hydrophones I, G, and H (which

% we will rename as #1, #2, and #3 here), for about 30 seconds, recorded

% with a DAT at Fs=48 kHz.

confirms previous work that identified two distinct boing vocalization types in the North Pacific. The eastern boing _n=22_ has a pulse repetition rate of 92 s−1 and a duration of 3.6 s         10,83 ms

The record says that the minke whale speak every 5 minutes.

Comparing

For example TK, that eliminate the signal!!!! It is not a delta like de sperm whale y se carga todo en MINKE

The delay was lowest!!! In our data the delay is very big, of 10s with 96 khz, alucina. GROUND TRTH WITH AUDCADICITI

| Click type | Centroid frequency (kHz) | Inter-click interval (s) | Duration of click (ms) | Duration of pulse (ms) | Range audible to sperm whale (km) |
| --- | --- | --- | --- | --- | --- |
| Usual | 15 | 0.5–1.0 | 15–30 | 0.1 | 16 |
| Creak | 15 | 0.005–0.1 | 0.1–5 | 0.1 | 6 |
| Coda | 5 | 0.1–0.5 | 35 | 0.5 | ~2 |
| Slow | 0.5 | 5–8 | 30 | 5 | 60 |

- TDE Algorithms (algorithms using delay between two sensors)

- Why two sensors?

- Better with more sensors?

In general, more sensors will lead to a higher robustness because of the redundancy. However, it should be pointed out that attention has to be paid to implementing the multichannel cross-correlation algorithm and the fusion method. Both need to synchronize either the signals observed at different sensors, or the cost functions from different sensor pairs. In case that the true delay is not the integral multiple of the sampling rate, we will have to either increase the sampling rate or use interpolation, which may significantly increase the computational complexity. In case that the observation signals or the cost functions are not properly aligned, we may not achieve much improvement.

All of the above methods can be improved by using several pairs of microphones, e.g., by hyperbolic curve fitting of multiple delay estimates11 or by linear 3-D intersection of multiple bearing lines.12–14

cc: Short Theory + Implementations (Problems and solution with the implementations and integration with real data)

The cross-correlation (CC) method is the most straightforward and the earliest developed TDE algorithm, which is formulated based on the single-path propagation model given in (1) with only two receivers, that is, N = 2. Suppose that we have a block of observation signals at time instant k,

xn[k] = _xn[0],xn[1], . . . ,xn[l], . . . ,xn[K − 1]_T

= _xn[k], xn[k + 1], . . . , xn[k + K − 1]_T,

(

where n = 0, 1 and K is the block size, then the delay estimate with the CC method is obtained as the lag time that maximizes the cross-correlation function (CCF) between two observation signals, that is,

_

τCC = argmax ΨCC[m],

where

ΨCC[m] = E	x0[l]x1[l + m]

is the CCF between x0[l] and x1[l], E{·} stands for the mathematical expectation, tC is an estimate of the true delay τ, m ∈ [−τmax, τmax], and τmax is the maximum possible delay.

Features

Easy implementation with the function of Matlab.

Low computational complexity

Non taking into account the channel response, only see for a correlation between signals.

We will see that after doing the noise reduction algorithms, it have a good performance in almost all the situations

gcc (phat and scot): Short Theory + Implementations (Problems and solution with the implementations and integration with real data)

The generalized cross-correlation (GCC) algorithm can be treated as an improved version of the CC method. Not only does it unify various correlation-based algorithms into one general framework, but it also provides a mechanism to incorporate

knowledge to improve the performance of TDE. This method has gained its great popularity since the landmark paper [5] was published by Knapp and Carter in 1976.

In this framework, the delay estimate is obtained as

τGCC = argmax ΨGCC[m],

ΨGCC[m] = Φ[k_]Sx0x1 [k_]ej2πmk_/K_

is so-called generalized cross-correlation function (GCCF), Sx0x1 [k_] = E{X0[k_]X∗ 1 [k_]} is the cross-spectrum,

(·)∗ denotes the complex conjugate operator, Xn[k_] is the discrete Fourier transform(DFT) of xn[k], Φ[k_] is a weighting function (sometimes called a prefilter), K_ is the length of the DFT, and σx0x1 [k_] = Φ[k_]Sx0x1 [k_] is the weighted crossspectrum. In a practical system, the cross-spectrum Sx0x1 [k_] has to be estimated, which is normally achieved by replacing the expected value by its instantaneous value, that is, Sx0x1 [k_] = X0[k_]X∗1 [k_].

There is a number of member algorithms in the GCC family depending on how the weighting function Φ[k_] is selected. Commonly used weighting functions include the constant weighting (in this case, the GCC becomes a frequencydomain implementation of the cross-correlation method shown in (9)), the smoothed coherence transform (SCOT) [44], the Roth processor [45], the Echart filter [5], the phase transform (PHAT), the maximum-likelihood (ML) processor [5], the Hassab-Boucher transform [18], and so forth. Combination of some of these functions is also reported in

use [46]. Another example is the ML processor with which the delay estimate obtained in the ideal propagation situation is optimal from a statistical point of view since the estimation variance can achieve the Cram`er-Rao lower bound (CRLB). It should be pointed out that in order for the ML processor to achieve the optimal performance, the observation sample space has to be large enough; the environments should be free of reverberation; the delay has to be constant; and the observation signals should be stationary processes. In addition, the spectra of noise signals have to be known a priori. If any of these conditions does satisfy, the ML algorithm will then become suboptimal, like other GCC members.

Taken by Marques book. We have used the phat:

For example, the PHAT algorithm uses ΦPHAT[k_] = 1/|Sx0x1 [k_]|.

Substituting ΦPHAT[k_] into (15) and neglecting noise effects, one can readily deduce that the weighted crossspectrum is free from the source signal and depends only on

the channel responses. Consequently the PHAT algorithm performs more consistently than many other GCC members when the characteristics of the source signal change over time. Different weighting functions possess different properties. It is also observed that the PHAT algorithm is more immune to reverberation than many other cross-correlationbased methods.

We implemented the Scot:

Another interesting weighting function is F( f ) 51/ASx1x1( f )Sx2x2

( f ). In this case, the corresponding normalized cross-spectrum Cc f ( f ) is simply the complex coherence function. We note that in the noiseless case, Cc f ( f )5Cpt( f ), which is not true, in general, in the presence of noise. The GCC methods can give good results when the reverberation of the room is not very high, but when the reverberation becomes important all of these techniques will fail because they are based on a simple signal model that does not represent reality.

LMS: Short Theory + Implementations (Problems and solution with the implementations and integration with real data). Remember to compare well LMS and AED. AED is indeed from 2000 by Benesty, but LMS for time-delay estimation is much older, see articles by Reed or Feintuch

This method, also based on the ideal propagation model with two sensors, was proposed by Reed et al. in 1981 [26]. It has been intensively investigated in the literature since Jingdong Chen et al. 5 then [28–30, 47]. Different from the cross-correlation-based approaches, this algorithm achieves time delay by minimizing the mean-square error between x0[k] and a filtered (FIR filter) version of x1[k], and the delay estimate is obtained as the lag time associated with the largest component of the FIR filter. If we define a signal vector of x1[k] at time instant k as x1[k] = _x1[k − L], x1[k − L + 1], . . . , x1[k], x1[k + 1], . . . , x1[k + L]_T and an FIR filter of length 2L + 1 as

h[k] = _h0, h1, . . . , hl , hl+1, . . . , h2L_T, (18) where L is the maximum possible time delay, then an error signal can be formulated as

e[k] = x0[k] − hT[k]x1[k]. (19)

An estimate of h[k] can be achieved by minimizing E{e2[k]} using either a batch or an adaptive algorithm. For example, with the least-mean-square (LMS) adaptive algorithm, h[k] can be estimated through

h[k + 1] = h[k] + μe[k]x1[k], (20)

where μ is a small positive adaptation step size. Given this estimate of h[k], the delay estimate can be determined as

τLMS = argmax IhI -M

Other adaptive algorithms [48] can also be used, which may lead to a better performance.

PROBLEMS IMPLEMENTATIONS: LENGTH OF DATA.

???

SOLUTION ???

AED: Short Theory + Implementations (Problems and solution with the implementations and integration with real data)

All the algorithms outlined in the previous sections achieve delay estimate by measuring the cross-correlation between two or among multiple channels. A common assumption with these methods is that each sensor receives only the direct-path signal. Recently, an adaptive eigenvalue decomposition (AED) algorithm was proposed to deal with TDE in room reverberant environment [15, 55]. Unlike the crosscorrelation- based methods, this algorithm first identifies the channel impulse responses from the source to the two sensors. The delay estimate is then determined by finding the direct paths from the two measured impulse responses. Apparently, this algorithm takes fully into account the reverberation effect during time delay estimation. For the signal model given in (5) with two sensors, if the noise term is neglected, one can easily check that x0[k] ∗ h1 = s[k] ∗ h0 ∗ h1 = x1[k] ∗ h0. (29)

Left multiplying (30) by x[n] and taking expectation yields

Ru = 0, (32)

where R = E{x[k]xT[k]} is the covariance matrix of the sensor signals. This implies that vector u which consists of two impulse responses is in the null space of R. More specifically, u is the eigenvector of R corresponding to the eigenvalue 0. It has been shown that the two channel impulse responses (i.e., h0 and h1) can be uniquely determined (up to a scale and a common delay) from (32) if the following two conditions hold [56–58]:

(i) the polynomials formed from h0 and h1 (i.e., the Ztransforms of h0 and h1) are coprime, or they do not share any common zeros;

(ii) the autocorrelation matrix of the source signal s[k], that is, Rss = E{s[k]sT[k]}, is of full rank.

See [56, 59] for a detailed description about the necessary and sufficient conditions for the identifiability. Note that the scale and common-delay ambiguities of blind identification techniques does not affect the problem of TDE. When an independent white noise signal is present on each sensor, it will regularize the covariance matrix; as a consequence, R does not have a zero eigenvalue anymore. In such a case, an estimate of the impulse responses can be achieved through the following algorithm, which is an adaptive way to find the eigenvector associated with the smallest eigenvalue Jingdong Chen et al. 7 of R [15]:

u[k + 1] = _u[k] − μe[k]x[k]

is an error signal, and μ, the adaptation step, is a positive constant. With the identified impulse responses_ h0 and_ h1, the time delay estimate is determined as the difference between two direct paths, that is

τAED = argmax IhI-IhI

Problems

Algotirhm of class but I cannot modified it to minke whales

Problems: distance between sensors and Fs.

The algorithm are not tracking the time delay estimation well.

NO SOLUTION

Algorithms used to reduce the noise

(Introduction of why we have to implement this algorithms)(Little preview of the First error simulations).  For each algorithm please explain what they are actually doing and try to compare them. you should measure the change in SNR to have an objective value of the enhancement

Filter: Short Theory + Implementations (Problems and solution with the implementations and integration with real data)

Filter pass band between 1-12 Khz. Flat in the band!

SNR data

mere filter

FOTO band

We have used it after the other methods.

Time Gain Normalization:  Short Theory + Implementations (Problems and solution with the implementations and integration with real data)

DIAPO

SNR

FOTO SPECTROGRAM Y SIGNAL DESPUES

Reduce the total amplitude of the data, doing first a smooth of the signal taking into account the preovious averages. We have put alpha to 0,9. This leads to a very flat data. Then, we nomalize the input by the estimation of the long term average level in the preovuis time.

THe signal is reduced, but the noise more.

THe first code was not doing, but after a few tries, we have done a full code. It is not working well.

SPECTROGRAM: why feo. It has values near 0 in all the situations, that is why we have a this kind of specgram. But we can see here where the signal is. O 0's o infinitos, then the spectrogram is really bad (Also the randomness of the remain signal)

Percentile Noise Removal: Short Theory + Implementations (Problems and not at all solutions with the implementations and integration with real data)

DIAPO

FOTO DEL PERCENTILE A COGER.

In the frequency domain, we calculate the estimate noise doing the percentile. Then, we substract it form the spectrogram of all the signal. We put an desto pa que no suba de 0.

Recta de los numeros

FOTOS THE COMO QUEDA LA SEÑAL Y EL ESPECTROGRAMA

SNR

Hard subtraction of noise! after proving, this is the best numbers,! more c will destroy the specgram and the signal!less it could not be sufficient to eliminate the noise, although the specgram is cleaner.

best values obtained in all the things

PROBLEMS: we have problems with the estimating of the noise model, (windows elected, time shift)  but thanks to ludwig and the code of Climent qbout spectral subtraction, we achieve a more or less PNR with a very good results in simulation despite the fact that de spectrogram is not good. due to 0!

I t could be improved looking for a better window length, window shift and values of the constant, but we have not time to improve it(to have a better specgram basucally)

Spectral subtraction: Theory + Implementations (Problems and solution with the implementations and integration with real data). Try to explain the relationship between spectral substraction and Percentile Noise

FOTOS

SNR

DIAPOS CLIMENT! not problems

We can see the relationship between the percentile and spectrtal. If we use the percentile 50 (median filter), there ius like a spectral subtraction doing an average of the noise spectrum.

CODE CLIMENT

Better secgram, less snr. lmore smooth the signal. Similar results in tde

Entropy in the asses

The entropy is a bit of a tricky measure, it is good if you can explain that in the presentation, you can criticize the choice of entropy for the measurement of noise reduction, it gives you only a partial idea of what happens. Basically it is considered that entropy measures randomness or a quantity of information.  We expect that with noise reduction the noise is reduced and that the noise is random but indeed noise can be correlated and sometimes you actually remove a lot of noise but add some randomness to the graph by making some denoising, so in some cases the entropy can increase instead of decreasing even though you have denoised ! In general the spectrograms show this well.

Simulations

- Explain the procedure and the GUI implemented to facilitate the simulations

- Times choosing and what kind of events (1-Without interference 2- With Interference)

- Sensors elected to do the simulations

First we always did the simulations with the test signals. With chirp and seno and noise delayed. Then, when the algoruothm works well with this signals, we pass to do the simulations with real data.

DATA 1

First we load two signal and cut it to focus in the events we like. Knowing that the first event of the sensor 1 is in 2:35, and knowing the position of the sensors, we choose a window of signal of 2*MAXTDOA. Due the distance betwen sensors, is a large window, near 20-25 s. This would be a problem to do adaptive algorithms.

Then, after cut the signal, we pre-proccessed it with a noise removal algorithm. Finally, we do the TDOA with the two signals. Knowng the true delay, we only have to do a ground truth assess. Then, we record the reuslts with the relative error and error samples-

.!   We have done the simulations with sensors 1,2,3,4 and 5 with an interesting results. Because is a good representative sample with all the possible positions of the sensors.

First event: 2:20-2:45    No interference, only noiseScond event: 8:40-9   With interference of other animals (dolphins and whales) For this reason, this is more critical and is more important to reduce well the noise and interference.

GUI implementation and how works????

The Graphical User Interface, built using the GUIDE MATLAB tool, has all the capabilities stated at the Introduction, section B, ”Scope of the project”. An screenshot of the same can be seen at Figure 2 using the GCC-SCOT result with Time Gain Normalization. The two loaded signals come from sensors 1 and 2 and are focused at an event of interest (a Minke whale sound).

The 3D distribution of the PMRF sensors can be seen and rotated using the option View 3D position of sensors. A screenshot is attached at Figure 3.

The code of the GUI is at Annex C (interactive TDE.m). In addition, a handy straight-forward GUI has been built in order to rapidly compare the algorithms and assess its pros and cons on each kind of signal. Such GUI allows to visually:

_ Import a pair of raw .wav signals

_ Select a time segment of this signals in minutes and

seconds

_ Plot their time evolution or spectrogram

_ Apply any combination of the preprocessing methods

_ Choose the desired TDE algorithms to assess

_ Visualize the 3D disposition of the underwater sensors

- Results without pre-processing

- cc,gcc with test signals------ Photo  :)

- cc, gcc with real data-------- Photo :(   Tell why it doesn't work and explain the solutions

Fotos y que va mal por culpa del alto ruido y que encima esta correlado, y entonces los algoritmos de cross correlation detectan el ruido y no la señal. Por esta razon, el delay calculado es casi 0 entre los sensores.

Solutions, pre-process the signal with the algorithms stated before.

With Time gain, Filter + Time Gain, PNR, Filter+ PNR, Spectral subtraction, Filter+ Spectral subtraction preprocessing algorithms

- gcc, cc, AED, LMS

- Photos (Correlations)

- Data of the results with ground truth (%relative error and error samples compared to the Real delay samples)

- Interpretation of the results (What is wrong? Why some algorithms do not estimate the signal correctly? Solutions

- Best Algorithm and why, Worst algorithm and why, These results agree with the expected behavior of the algorithms? (Theoretical)(Why?)

- Comparison of cost of calculation. It is relevant?

- Demonstration of our procedure of estimation with the GUI on live!

Xavi:  sensors 1-2     event 1  filter + Percentile noise removal and Scot

1-2                     event 1    Spectral subtraction and phat and Scot

Phat didn't work because is more for room acoustics and the features of inderwater is bad for phat.   basically when you have sensors that are far from each other the coherence is low and SCOT manages to take it into account. THEN IT LEADS to a 0 delay and a bad implementation of the method depends on the pre-whitening method you use. As we said, phat is the algorithns that works worst.

Good results as we said before!!

- Localization of the Whales

- The way we use to localize the whale from the previous results of Time Delay estimation:

- What kind of algorithms

- What kind of code

- With how many sensors

- Results (Are they agree with the real localization of the whales and with the TDE? Why? Why not?)

*Show the graphic

for tracking a sperm whale on a 2D map. We develop a rudimentary multilateration system, on the basis of 2 TDOAs only.

We first show the position of the hydrophones on the map (Fig. 6.27), and then plot the hyperbolas corresponding to each pair of TDOA, assum-ing the speed of sound in water is 1510 m/s.

Multilateration is a navigation technique based on the measurement of the difference in distance to two or more stations at known locations that broadcast signals at known times. Unlike measurements of absolute distance or angle, measuring the difference in distance results in an infinite number of locations that satisfy the measurement. When these possible locations are plotted, they form a hyperbolic curve. To locate the exact location along that curve, a second measurement is taken to a different pair of stations to produce a second curve, which intersects with the first. When the two are compared, a small number of possible locations are revealed, producing a "fix".

Multilateration is a common technique in radio navigation systems, where it is known as hyperbolic navigation. These systems are relatively easy to construct as there is no need for a common clock, and the difference in the signal timing can be measured visibly using an oscilloscope. This formed the basis of a number of widely used navigation systems starting in World War II with the British Gee system and several similar systems introduced over the next few decades. The introduction of the microprocessor greatly simplified operation, greatly increasing popularity during the 1980s. The most popular hyperbolic navigation system was LORAN-C, which was used around the world until the system was shut down in 2010. Other systems continue to be used, but the widespread use of satellite navigation systems like GPS have made these systems largely redundant.

Multilateration should not be confused with trilateration, which uses distances or absolute measurements of time-of-flight from three or more sites, or with triangulation, which uses the measurement of absolute angles. Both of these systems are also commonly used with radio navigation systems; trilateration is the basis of GPS.

Other: In trigonometry and geometry, triangulation is the process of determining the location of a point by measuring angles to it from known points at either end of a fixed baseline, rather than measuring distances to the point directly (trilateration). The point can then be fixed as the third point of a triangle with one known side and two known angles.

FOTO

METHOD! PERCENTILE + XCORR!!  BEST RESULTS!  It is quite normal that the curves don't interesct all in one point, but we can see where it is more or less. The first 5 curves have a time delay estimation less 1%. But 1% of 12 km is 120m, that is why they curves don't match in one point.

The other tweo curves are less accurate, it could be because the tdoa is a little worst, aorund 1-2% it could be (no emos exo simulaciones)

Podriamos aver usado otros algoritmos, pero poco tiempo y este ya encontrava bien la ballena

(POSICIONDE LA BALLENA)

At least two pairs of mics are required for multilateralization, i.e. 3 mics

For 2D multilateralization from intersection of loci we need to solve a system of two non-linear equations

The third TDOA may not have the same intersection

Multiple TDOA/DOA estimates lead to a minimization problem of an over-determined error function E(s) of position s:

Direct recursive algorithms are possible (gradient descent) but present some problems: initial estimate, instability, local minimum...

Closed-form approximations are computationally practical and perform well in easy scenarios. However, difficulties in very high noise and reverberant scenarios and the inability to work on multi-source situations are the main drawbacks. Main methods: crossing lines (CL), steered-response power (SRP)

DIAPOS (TWO OTHER METHODS WITH MORE PAIRS OF MICROFONOS) CONTRAS Y FAVOR

Interpretacion: Tiene logica si miramos los TIME DELAY REALES Y LA POSICION DE LOS SENSORES! POR LO QUE EL TDE CON PERCENTILE I XCORR Y MULTIALLENCION A SIDO BUENO! Aunque SE PUEDE MEJORAR CON OTRAS TECNICAS DE LOCALIZATION Y CON ALGORITMOS DE TDE MAS POTENTES COMO EL EIGENVALUE (NO EMOS PODIDO)

FOTOS Y GRAFICA!

Conclusion

- Main problems (code), why? how to fix it?

- Best Noise reduction algorithms? Why?

- Best TDE algorithms?  Why?

- Worst TDE Algorithms? Why?

- Goals achieved?

Cons to face: The passive TDOA approach and the underwater environment imply some inconvenients that must be taken into consideration:

1) The incoming signal is totally unknown, in contrast of in active (TOA) approach, where a simple matched filter would be optimum when the reference signal is produced by us. In our case, TDOA can do no presumption of the waveform, excluding the frequency range. Minke whales usally emit sounds from 1 to 12 kHz.

2) Signal to noise ratio (SNR) is very high. This will be the main limitation and that’s the reason of the emphasis on the prefiltering methods for noise reduction.

3) Correlated noise, as the main contribution to it is not white noise, but strong interferences from other animals, devices and water currents.

4) Reflections on the water surface, mainly. This can make the algorithm to get confused by them and could return a wrong result. Reflections on the seafloor are negligible, as hydrophones are usually placed very near to the ground.

5) The recordings will not be just a shifted version of the other ones. Noise, reflections and interferences will introduce huge bias and variance in the estimation if not treated correctly.

6) Sources (whales) will not be still, we need an adaptive method that track the whale

Future Work

- Improve the code implementation of:

- AED, LMS, PNR, Time Gain

- Compare and enforce the Localization of the Whale using TDOA with:

- Entropy Algorithm

- Algorithms using more than 2 sensors, you already combine more than two sensors but indeed you could make it using all the pairs jointly.

- Tracking of the whale while it is moving. In our case yes you would need a specific tracker.