The actual problem

The solution

- Estimate the whale localization by TDOA

    - What is TDOA?

    - Sensors used?

    - Necessity of noise reduction?

    - Actual algorithms?

There are two general groups of methods for the localization of sources : TDOA-based methods and Space-time methods (~ beamforming). In the second type of methods based on spatial filtering, only one step is required but those methods are more computationally demanding since they require to scan the whole space. The first type of methods is more general and deals with a very simple yet a key problem for signal processing and estimation theory: time-delay estimation. In these methods time-delay estimation is performed and then a time-space inversion constructs a position based on the estimated time-delays.

TDOA endeavors to determine the travel time of a wavefront between two spatially separated receiving sensors, which is often of concern to a passive system such as passive sonars and microphone array systems. Although there exists intrinsic relationship between the TOA and TDOA estimation, their essential difference is literally profound.

TDOA is often acquired by comparing the signals received at two (or more) spatially separated sensors. It requires mainly a two step method. In the first step, the time differences of arrivals of the signal of interest to the different sensors are measured. This can be done by cross-correlation and peak detection of the sensors’ signals. Many methods have been presented that deal with enhancing the correlation result, among them [1],[2]. In the second step, the measured TDOAs are used to solve the hyperbolic system.

The estimation of TDOA would be an easy task if the two received signals were merely a delayed and scaled version of each other. In reality, however, the source signal is generally immersed in ambient noise since we are living in a natural environment where the existence of noise is inevitable. Furthermore, each observation signal may contain multiple attenuated and delayed replicas of the source signal due to reflections from boundaries and objects. This multipath propagation effect introduces echoes and spectral distortions into the observation signal, termed as reverberation, which severely deteriorates the source signal. In addition, the source of the wavefront may also move from time to time, resulting in a changing time delay. All these factors make time delay estimation a complicated and challenging problem.

The improvements are achieved through three different ways:

- The first is to incorporate some a priori knowledge about the distortion sources into the GCC method to ameliorate its performance.

- The second is to use multiple (more than two) sensors and take advantage of the redundancy to enhance the delay estimate between the two selected sensors.

- The third is to take into account of reverberation in the signal model and exploit the advanced system identification techniques to improve TDE.

Two or more sensors:  Two more efficient and there are algorithms good for the TDE.

Other with more sensors like: Fusion algorithm based on multiple sensor pairs ^Priori knolgede

Multichannel cross-correlation algorithm

Adaptive multichannel time delay estimation

cc,gcc,

LMS, AED,

Noise reduction

NOISE REDUCTION BECAUSE IN WATER THE SIGNAL IS HIGLY CORRELATED AND WITH A HIGHT LEVEL. WE NEED TO ELIMINATE THAT, because in the algorithms we will not be able to find the true delay because of noise.

- si es blanco te jode el conocer el channel

- si es correlado y quieres usar metodos de correlacion, siempre detectaras el ruido y no la señal

Therefore, we will use Noise algorithms to:

-Reduce noise and interference :   make signals more evident     symplify detection

- put signal into standarised form    easy to set standarised threshold    well SNR  low background noise level

Benefits of TDOA vs Beamforming in the underwater environment.

- Why?

Particularities of underwater environment

Beamformer

A beamformer is steered to all directions in order to compute the spatial power, assuming that the power from the source direction is high. So, directions that  the output power are the estimated locations.

Often the maximization function does not present strong global peaks . The methods with better performance are computational expensive

Most of these methods are not designed for arbitrary array geometries  Assume narrow-band signals, while audio is a broadband signa

WE USE TDOA BECAUSE THE DISTANCE OF THE SENSORS. IF WE WOULD HAVE USED BEAMFORMING, WE had seen SPATIAL ALIASING

TDOA

Large distance between sensors. It is more robust to bad calibration (amplitude, sensor position).

Simplicity, robustness to reverberation and absence of model assumptions. Main drawback: multi-source TDOA is difficult

This is the only feasible method for this case, as beamforming cannot be used when distance between sensors is large.

Deepen the goals

- Implement and compare algorithms (95% of the spent time)

First, read the papers about the different time-delay estimation algorithms in different environments and papers about Minke Whales and their kind of sounds.

Rankin_JASA05_Minke_Boing-----Minke whales paper

tde_ IN ROOM-------- all the algorithms performance in room

On using the LMS on tde---- LMS

shipstrike-------     Collisions between whales and ships

eigenvalue decomposition algorithm-----------AED vs GCC

Tde using LMS--------------------LMS

Collisions between vessels and whales

whale spotting ---------- TDE theory and code with Matlab

- Deal with real data

- Minke whales (History and curiosities of Minke whales and the sound characteristics)

The “boing” sound was first described by Wenz _1964_ from U.S. Navy submarine recordings made in the 1950s off San Diego, California, and Kaneohe, Hawaii. Despite much attention, the source of the sound has remained a mystery until now.

Boings were detected during three cetacean research surveys, the 1997 Sperm Whale Abundance and Population Structure Survey _SWAPS_, the 2002 HICEAS cruise, and the 2003 Stenella Abundance Research Survey _STAR_.

During a recent cetacean survey of the U.S. waters surrounding the Hawaiian Islands, the probable source of the mysterious “boing” sound of the North Pacific Ocean was identified as a minke whale, Balaenoptera acutorostrata. Examination of boing vocalizations from three research surveys confirms previous work that identified two distinct boing vocalization types in the North Pacific. The eastern boing has a pulse repetition rate of 92 s−1 and a duration of 3.6 s and was found only east of 138°W.  This match with the 1964. The central boing has a pulse repetition rate of 115 s−1 and a duration of approximately 2.6 s and was found only west of 135°W.

**Pulse repetition rates of the eastern and central boings were distinct.

Boing vocalizations consist of a brief pulse followed by a long call that is both frequency modulated _FM_ and amplitude modulated _AM_ _Figs. 2 and 3_. Based on our measurements of 128 boings, the calls can be grouped into two distinct call types with nonoverlapping pulse repetition

Minke whales are the smallest of baleen whales and are typically encountered individually or in small groups of two or three. They have inconspicuous blows, and do not surface for extended periods of time. Additionally, the waters covered by these surveys are dominated by high sea states associated with the trade winds. Combined, these features reduce the probability of sighting minke whales, and may explain the discrepancy between the low visual detection of minke whales and the high acoustic detection of boings. The swimming speed and direction of the minke whale during this encounter _sighting number 267_ was found to be nearly identical based on the visual and acoustic detections _5.7 km/h and 5.6 km/h, respectively_. This speed is reasonable, but higher than that found by other researchers

Our Real Data provided by Ludwig  : describe data where does it comes from, why is this case particular compared to the data we have used in class ( species, distance between sensors, duration od signals and distance between minke whales sounds compared to sperm whales, etc)

- Duration of the data-

- Number of sensors-

- Events taken into account

- True delay of the two events

- Sampling rate  (Enunciate the future problems with LMS and AED due to the Hight sampling rate, the problem is not really the sampling rate (it's better to have more samples !) but the distance between the sensors)-

Data and information is coming from PMRF,  an instrumented US Navy testing range located off the island of Kauai, Hawaii. Data were collected from seven bottom mounted hydrophones (4 to 5 m off the seafloor) in deep water (nominally 4,600 meters) approximately 45 km northwest of Kauai. The relative location of the hydrophones, their designations and filenames for the data files are as shown in Table 1.

With this dataset you’ll be able to make localization of minke whales (Balaenoptera acutorostrata) based on their boings sounds.. Beware that vocalizations from other species are also present,

The dataset is from the 27th of April 2009 at approximately 12:00 Hawaiian standard time. Thirty minutes of data from each hydrophone are provided as three files (approx. 10 minutes per file). The sampling rate of the recordings (windows PCM wav format) is 96 kHz with 16 bits resolution. The data are sampled simultaneously, so sample N from one file is the same relative time as sample N of a second file with the same filename. The file format is .wav in little-endian format.

Distance

Distance between sensors, 12-27 km. This leads to a difficulty to implement and work with LMS and AED due the big maximum time delay between sensors.  This leads to have a big big filter length and added to a big sampling rate, its becomes a difficult time delay estimation with our computers due the computational calculs. Besides, the PHAT doesn't work because of the low coherence between signals.

Events taken

We focus on two events of the first 10m of recording.  Around the minut 2:30 and the minut 8:50.  The treu delay varies, but looking into Audacity we have calculated it. It varies from 1s to 12 s between the seven sensors in the two events. The delay between the two events vary, this would explain the whale is moving.

Sperm

The difference with sperm whales, the sounds we have used in class, is that the sperm whales have a spectrogram with cliks, little impulses like deltas. En cambio, las minke whales had other sound, with more high frequency. The sounds are not clicks, and therefore we have more difficulties to calculate de tde.

sperm ::: The distance between most hydrophones is 5 nautical miles (about 7.5 km).

% We will work with the sounds received by hydrophones I, G, and H (which

% we will rename as #1, #2, and #3 here), for about 30 seconds, recorded

% with a DAT at Fs=48 kHz.

confirms previous work that identified two distinct boing vocalization types in the North Pacific. The eastern boing _n=22_ has a pulse repetition rate of 92 s−1 and a duration of 3.6 s         10,83 ms

The record says that the minke whale speak every 5 minutes.

Comparing

For example TK, that eliminate the signal!!!! It is not a delta like de sperm whale y se carga todo en MINKE

The delay was lowest!!! In our data the delay is very big, of 10s with 96 khz, alucina. GROUND TRTH WITH AUDCADICITI

| Click type | Centroid frequency (kHz) | Inter-click interval (s) | Duration of click (ms) | Duration of pulse (ms) | Range audible to sperm whale (km) |
| --- | --- | --- | --- | --- | --- |
| Usual | 15 | 0.5–1.0 | 15–30 | 0.1 | 16 |
| Creak | 15 | 0.005–0.1 | 0.1–5 | 0.1 | 6 |
| Coda | 5 | 0.1–0.5 | 35 | 0.5 | ~2 |
| Slow | 0.5 | 5–8 | 30 | 5 | 60 |

AED: Short Theory + Implementations (Problems and solution with the implementations and integration with real data)

All the algorithms outlined in the previous sections achieve delay estimate by measuring the cross-correlation between two or among multiple channels. A common assumption with these methods is that each sensor receives only the direct-path signal. Recently, an adaptive eigenvalue decomposition (AED) algorithm was proposed to deal with TDE in room reverberant environment [15, 55]. Unlike the crosscorrelation- based methods, this algorithm first identifies the channel impulse responses from the source to the two sensors. The delay estimate is then determined by finding the direct paths from the two measured impulse responses. Apparently, this algorithm takes fully into account the reverberation effect during time delay estimation. For the signal model given in (5) with two sensors, if the noise term is neglected, one can easily check that x0[k] ∗ h1 = s[k] ∗ h0 ∗ h1 = x1[k] ∗ h0. (29)

Left multiplying (30) by x[n] and taking expectation yields

Ru = 0, (32)

where R = E{x[k]xT[k]} is the covariance matrix of the sensor signals. This implies that vector u which consists of two impulse responses is in the null space of R. More specifically, u is the eigenvector of R corresponding to the eigenvalue 0. It has been shown that the two channel impulse responses (i.e., h0 and h1) can be uniquely determined (up to a scale and a common delay) from (32) if the following two conditions hold [56–58]:

(i) the polynomials formed from h0 and h1 (i.e., the Ztransforms of h0 and h1) are coprime, or they do not share any common zeros;

(ii) the autocorrelation matrix of the source signal s[k], that is, Rss = E{s[k]sT[k]}, is of full rank.

See [56, 59] for a detailed description about the necessary and sufficient conditions for the identifiability.

When an independent white noise signal is present on each sensor, it will regularize the covariance matrix; as a consequence, R does not have a zero eigenvalue anymore. In such a case, an estimate of the impulse responses can be achieved through the following algorithm, which is an adaptive way to find the eigenvector associated with the smallest eigenvalue Jingdong Chen et al. 7 of R [15]:

u[k + 1] = _u[k] − μe[k]x[k]

is an error signal, and μ, the adaptation step, is a positive constant. With the identified impulse responses_ h0 and_ h1, the time delay estimate is determined as the difference between two direct paths, that is

τAED = argmax IhI-IhI

Problems

Algotirhm of class but I cannot modified it to minke whales

Problems: distance between sensors

The algorithm are tracking all.

Algorithms used to reduce the noise

(Introduction of why we have to implement this algorithms)(Little preview of the First error simulations).  For each algorithm please explain what they are actually doing and try to compare them. you should measure the change in SNR to have an objective value of the enhancement

Filter: Short Theory + Implementations (Problems and solution with the implementations and integration with real data)

Filter pass band between 1-12 Khz. Flat in the band!

SNR data

mere filter

FOTO band

We have used it after the other methods.

Time Gain Normalization:  Short Theory + Implementations (Problems and solution with the implementations and integration with real data)

DIAPO

SNR

FOTO SPECTROGRAM Y SIGNAL DESPUES

Reduce the total amplitude of the data, doing first a smooth of the signal taking into account the preovious averages. We have put alpha to 0,9. This leads to a very flat data. Then, we nomalize the input by the estimation of the long term average level in the preovuis time.

THe signal is reduced, but the noise more.

THe first code was not doing, but after a few tries, we have done a full code. It is not working well.

SPECTROGRAM: why feo. It has values near 0 in all the situations, that is why we have a this kind of specgram. But we can see here where the signal is. O 0's o infinitos, then the spectrogram is really bad (Also the randomness of the remain signal)

Percentile Noise Removal: Short Theory + Implementations (Problems and not at all solutions with the implementations and integration with real data)

DIAPO

FOTO DEL PERCENTILE A COGER.

In the frequency domain, we calculate the estimate noise doing the percentile. Then, we substract it form the spectrogram of all the signal. We put an desto pa que no suba de 0.

Recta de los numeros

FOTOS THE COMO QUEDA LA SEÑAL Y EL ESPECTROGRAMA

SNR

Hard subtraction of noise! after proving, this is the best numbers,! more c will destroy the specgram and the signal!less it could not be sufficient to eliminate the noise, although the specgram is cleaner.

best values obtained in all the things

PROBLEMS: we have problems with the estimating of the noise model, (windows elected, time shift)  but thanks to ludwig and the code of Climent qbout spectral subtraction, we achieve a more or less PNR with a very good results in simulation despite the fact that de spectrogram is not good. due to 0!

I t could be improved looking for a better window length, window shift and values of the constant, but we have not time to improve it(to have a better specgram basucally)

Spectral subtraction: Theory + Implementations (Problems and solution with the implementations and integration with real data). Try to explain the relationship between spectral substraction and Percentile Noise

FOTOS

SNR

DIAPOS CLIMENT! not problems

We can see the relationship between the percentile and spectrtal. If we use the percentile 50 (median filter), there ius like a spectral subtraction doing an average of the noise spectrum.

CODE CLIMENT

Better secgram, less snr. lmore smooth the signal. Similar results in tde

Entropy in the asses

The entropy is a bit of a tricky measure, it is good if you can explain that in the presentation, you can criticize the choice of entropy for the measurement of noise reduction, it gives you only a partial idea of what happens. Basically it is considered that entropy measures randomness or a quantity of information.

We expect that with noise reduction the noise is reduced and that the noise is random but indeed noise can be correlated and sometimes you actually remove a lot of noise but add some randomness to the graph by making some denoising, so in some cases the entropy can increase instead of decreasing even though you have denoised ! In general the spectrograms show this well.

With Time gain, Filter + Time Gain, PNR, Filter+ PNR, Spectral subtraction, Filter+ Spectral subtraction preprocessing algorithms

- gcc, cc, AED,

- Photos (Correlations)

- Data of the results with ground truth (%relative error and error samples compared to the Real delay samples)

- Interpretation of the results (What is wrong? Why some algorithms do not estimate the signal correctly? Solutions

- Best Algorithm and why, Worst algorithm and why, These results agree with the expected behavior of the algorithms? (Theoretical)(Why?)

- Comparison of cost of calculation. It is relevant?

Phat didn't work because is more for room acoustics and the features of inderwater is bad for phat.   basically when you have sensors that are far from each other the coherence is low and SCOT manages to take it into account. THEN IT LEADS to a 0 delay and a bad implementation of the method depends on the pre-whitening method you use. As we said, phat is the algorithns that works worst.

Good results as we said before!!

Conclusion

- Main problems (code), why? how to fix it?

- Best Noise reduction algorithms? Why?

- Best TDE algorithms?  Why?

- Worst TDE Algorithms? Why?

- Goals achieved?

Cons to face: The passive TDOA approach and the underwater environment imply some inconvenients that must be taken into consideration:

1) The incoming signal is totally unknown, in contrast of in active (TOA) approach, where a simple matched filter would be optimum when the reference signal is produced by us. In our case, TDOA can do no presumption of the waveform, excluding the frequency range. Minke whales usally emit sounds from 1 to 12 kHz.

2) Signal to noise ratio (SNR) is very high. This will be the main limitation and that’s the reason of the emphasis on the prefiltering methods for noise reduction.

3) Correlated noise, as the main contribution to it is not white noise, but strong interferences from other animals, devices and water currents.

4) Reflections on the water surface, mainly. This can make the algorithm to get confused by them and could return a wrong result. Reflections on the seafloor are negligible, as hydrophones are usually placed very near to the ground.

5) The recordings will not be just a shifted version of the other ones. Noise, reflections and interferences will introduce huge bias and variance in the estimation if not treated correctly.

6) Sources (whales) will not be still, we need an adaptive method that track the whale